# Mechatronics Lab Autonomous Sumo Competition – Instructions & Requirements

# Objective

Build an autonomous robot on the Arduino Alvik platform that:

- Navigates a 2-meter diameter arena.

- Competes against 4–6 other robots.

- Uses onboard sensors and actuators to capture and hold the central hill (marked red/blue) or remain as the last operational robot within the arena.

Competition Duration: 10 minutes

# Technical Requirements

- Autonomy

  - No manual control during matches.

  - The robot must start, navigate, and decide actions autonomously.

- Sensors - You must collect and log data from at least:

  - Encoders (for odometry).

  - IMU (orientation).

  - Distance or line sensors (for arena edges).

- Data Logging - During each match, log:

  - Time-stamped sensor data.

  - Actuator commands.

  - State transitions (e.g., searching, attacking, defending).

  - Use wireless logging (using Wi-Fi scripts from Lab 5) to keep the robot untethered.

- Arena

  - Arena floor: 2 meters diameter.

  - Border: marked by black tape or 3D printed tilted surfaces to detect edges.

  - Center hill: small raised area painted red/blue.

# Scoring

  - Primary goal: Be on the hill at the match end.

  - Secondary goal: Be the last robot operating inside the arena.

  - Bonus Points:

    - High-quality plots and analysis of your sensor data.

    - Trajectory reconstruction using wheel odometry and a 2-DOF differential drive kinematic model

# Report Deliverables

Each team must submit a concise technical report containing:

- Block Diagram

  - Show the software architecture and sensor/actuator connections.

- Code Overview

  - 1–2 pages of commented code snippets demonstrating:

    - Autonomous state machine logic.

    - Sensor data acquisition.

    - Actuator control.

- Tactics Explanation

  - ½ page describing your robot’s strategy:

    - How you search for opponents.

    - How you avoid falling out.

    - How you attempt to capture the hill.

- Data Plots

  - Time plots of:

    - Encoder counts / Wheel position.

    - IMU angles.

    - Distance sensor readings.

  - Bonus points, include a 2D plot of the estimated trajectory from odometry.

- Concluding Experience

  - ½ page reflection discussing:

    - Challenges encountered.

    - Insights gained.

    - What you would improve.